# Eyal Espresso Requirements and Design

## Document Status: Working Baseline

## Version: 0.2.1

## Owner: Eyal / Codex

## Reviewed on: 08-Mar-26 08:51:43

# Product Overview

Purpose: Build an ESP32-S3 based espresso-machine UI and control application for the Waveshare 4.3 inch touch LCD hardware.

Problem Statement: The application must provide a stable, touch-friendly machine workflow UI while preserving reliable RGB panel behavior on the target hardware.

Success Criteria: Stable display, responsive touch interaction, clear workflow navigation, and maintainable project documentation and revision tracking.

# 2. Users and Use Cases

Primary Users: Espresso machine owner/operator, firmware developer, and tester.

Key Use Cases: View machine status, start/stop brew, enable steam mode, switch brew profiles, adjust temperature, adjust preinfusion, and inspect live header metrics.

User Journey Notes: User lands on a dashboard, moves between tabs by touch or swipe, enters Brew for runtime control, Profiles for recipe selection, and Settings for tuning.

# 3. Functional Requirements

## General requirements

- There shall be 3 states to the system: Initialization, Use, and Error state

- The "Initialization" state shall have a splash screen with textual information about the progress

- The "Use" state shall be UI related and allow the user to select and operate the system

- The "Error" shall have a UI with the persistent errors displayed and a single button "Reset", leading to initialization state where the system would perform a BIT to test for status.

- The LCD backlight shall turn off automatically after 30 seconds in case no touch event was performed (Implemented: Yes)

## Initialization Requirements

- The initialization state shall start with a question to the user: Run in Offline Mode? With two buttons “Yes”, “No”

- In case the user runs the system in offline mode, the connection with the Gaggia server will be simulated as well and no real WiFi connection established (Implemented: No)

- During initialization, the client shall test for internal ESP32 components status, as well as request from the Server (i.e Gaggia Controller) to initialize the coffee machine and respond with its status. (Implemented: Yes)

- While initializing, the splash screen shall have a text â€œInitializing Systemâ€¦â€ as well as changing texts that indicate what component is getting initialized. (Implemented: Yes)

- In case a component has not succeeded to initialize, the text shall be highlighted red and a Fail indication on screen.  (Implemented: No)

- Upon failing initialization for a component, a button on the bottom named â€œConfirmâ€ appears and after pressed the screen will switch to the â€œErrorâ€ screen

- The Initialization shall initialize the following components in order:
the ESP32 client and all it’s internal hardware, the WIFI connection to the Gaggia Server, the SystemConstants DB, the Gaggia BIT and status check, Version Compatibility Check with Server.

- The Initialization shall apply all constants from the SystemConstants DB to the client (Implemented: No)

## UI related requirements

- The UI shall provide four main tabs named:

Brew, Realtime, Profiles and Settings. (Implemented: No)

- The UI shall support left and right swipe gestures to change the active tab. (Implemented: Yes)

- The Home tab shall show the active profile and target temperature. (Implemented: Yes)

- The Brew tab shall expose Brew/Pump and Steam mode controls. (Implemented: Yes)

- Enabling Brew shall reset shot time and clear Steam mode. (Implemented: Yes)

- Enabling Steam mode shall clear Brew mode. (Implemented: Yes)

- The Profiles tab shall allow one-touch selection of profiles from the list provided by the DB

## Settings Tab

- The Settings tab shall allow tuning target temperature in the range 86 C to 98 C. (Implemented: Yes)

- The Settings tab shall allow tuning preinfusion time in the range 0 s to 12 s. (Implemented: Yes)

- The header shall continuously display operating mode, active profile, target temperature, shot timer, and synthetic pressure feedback. (Implemented: Yes)

- Ther shall be a button on the Settings TAB named "Set Clock", this button will open a diffent UI with a "Done" button at the bottom and scroll widgets to select day, month, year, hour, minutes. Upon pressing "Done" the RTC will set a new time according to the user input (Implemented: No)

- There shall be a button on the Settings TAB, this will be "Connection Info" Button. when pressing it a new screen with connection information appears with the IP, port, and telemetry data as text, in this screen a "Done" button will be present at the bottom and pressing it will close this screen and go back to the main UI (Implemented: Yes)

- There shall be a button on the Settings TAB, this will be "System Constants" Button. when pressing it a new screen with system contacts information appears. In this screen a "Done" button will be present at the bottom and pressing it will close this screen and go back to the main UI (Implemented: Yes)

## Realtime Tab

- The Realtime tab shall include a plot area with multiple plots of data on a timescale + a textual information that is important about the current and last brew. (Implemented: No)

- The Realtime tab plot area shall include horizontal and vertical grids as well as X-axis ticks and labels below the graph and Y-ticks and labels from the right of the graph. (Implemented: No)

- The Realtime tab plot shall display Realtime data with a resolution 0.2 seconds. (Implemented: No)

- The Realtime tab plot shall display 2 graphs: Pressure and Flow (Implemented: No)

- The Realtime tab plot shall display with an auto-scale to Y-axis that normalizes the maximal height of the graph to the maximum of the data values and modifies the Y-Axis labels as well. (Implemented: No)

- The Realtime tab shall include a plot area with multiple plots of data on a timescale + a textual information that is important about the current and last brew. (Implemented: No)

## Data and storage requirements

- There shall be a database XML file on the system which contains the constants used by the system. This database shall be named: SystemConstants.xml and a default file stored in the project. This database will be referred to as "constants" in this document. (Implemented: No)

- The system shall read and load constants from the constants DB during initialization (Implemented: No)

- The constants shall include the profile names and the number of profiles. The project shall include default profiles: Classic 9 Bar, Turbo Shot, and Light Roast.

- The SystemConstants file shall include (at minimum):

Brew Profiles List

Brew Profile Targets for each Brew profile

Limitations on minimum and maximum for pressure, flow, and temperature during brewing

Connection details

Version number for the Client

Version number compatible with the client

# 4. Non-Functional Requirements

Performance: Screen transitions and control updates shall feel immediate on the ESP32-S3 target. (Implemented: Yes)

Reliability: The display path shall remain visually stable without right-shift drift or animation flicker under the validated RGB configuration. (Implemented: Yes)

Safety: Brew and Steam shall not remain active together in software state. (Implemented: Yes)

Maintainability: Requirements, revision history, defects, and workflow rules shall be tracked in repository documents. (Implemented: Yes)

# 5. UX and UI Design

Navigation Model: Top tab bar plus left/right swipe navigation across a tabview. (Implemented: Yes)

Visual Style: Dark theme with blue accent, muted text for secondary information, and card-based sections. (Implemented: Yes)

Interaction Notes: Use large touch targets, maintain clear selected-tab state, and keep active profile visually highlighted. (Implemented: Yes)

All the text and interface shall be under one of the tabs except for a clock which will be always displayed on the bottom 10% of the screen. The tabs shall occupy the top 90% of the screen with the tab names on the top side of the screen (Implemented: Yes)

The clock shall be in “dddddd, MMM-D, YYYY HH:MM:SS” format in the persistent clock bar.

Both press and toggle buttons shall have a button type of interface and press in/out animation and colors (Implemented: Yes)

The minimal distance between UI objects that are mot in the same group shall be 10 pixels, this applies to different buttons, context boxes and groups, but not to objects in the same group.

# 6. System Design

Architecture Summary: ESP-IDF application using LVGL 9.x, RGB panel output, and GT911 touch input on the Waveshare ESP32-S3 4.3B board.

Module Breakdown: main app entry, hardware initialization, LVGL port layer, and ui_screen UI composition/state logic.

Hardware Dependencies: Waveshare ESP32-S3-Touch-LCD-4.3B, RGB LCD, GT911 touch controller, ESP32-S3 with PSRAM.

External Interfaces: LVGL events, ESP-IDF display/touch drivers, serial logs, and local build/flash tooling.

# 7. Data and Configuration

Persistent Settings: Target temperature and preinfusion are current tunable values and are candidates for future persistence.

Profiles/Recipes: Current predefined profiles are Classic 9 Bar, Turbo Shot, and Light Roast.

Versioned Config Items: Display stability currently depends on `psram_trans_align = 64`, 10-line bounce buffer sizing, and `on_bounce_frame_finish` callback usage.

# 8. Validation and Test Plan

Unit Tests: Add targeted checks where practical for UI state transitions and helper scripts.

Hardware Verification: Rebuild, flash, monitor the first 5 seconds of logs, and visually validate display stability plus touch navigation.

Acceptance Tests: Confirm tab switching, swipe behavior, profile selection, brew/steam mutual exclusion, and settings adjustment on the target panel.

# 9. Risks and Open Questions

Known Risks: RGB timing/configuration changes can reintroduce drift, flicker, or black-screen regressions.

Open Questions: Which settings and profile data should be persisted, and what machine-control backend should be bound to the current UI mock controls.

Deferred Decisions: Final production information architecture, persistence format, and real sensor/control integration details.

# 10. Client-Server Interface Design

## General Information about the Interface Design

The Interface Design in this record is high level data oriented, this assumes that low level checksum and data integrity is in place and working as a part of the infrastructure.

## Keep Synchronization

The design shall include a full duplex handshake with incremental value response. This shall be done by having a local variable named “LiveInteger” at both sides of type uint that starts as zero during initialization and increments every incoming/outcoming message from the server to and from the client until returning to zero (no overflow) and starting again. Every message pass shall be tested for expected “LiveInteger” increase and if passed incremented for the reply. In case the expected value has failed the check, the system will move to the “Error State” and handle errors according to that state. This shall be a part of any message type herein.

An acknowledge message shall be sent from client to server for every message the server sends.

## Types of Messages

The interface shall include 2 types of messages:
1. Config MSG

- Data MSG

Each of them shall have a static structure, with at minimum: LiveInteger, MessageType, Timestamp

## Config MSG

## This shall have a at minimum: Set/Get configuration, data to be

# 10. Change Log

2026-03-07: Initial working requirements baseline created from current implemented UI and project rules.